Infrared transmission experiment

Introduction the devices

This time, we will introduce the infrared transmitter and receiver modules, which actually play an important role in our daily life. Now such a device is widely used in many home appliances, such as air conditioning, television, DVD, etc. It is based on wireless remote sensing, but also a remote control, it is necessary to study its principle and how to use.

Infrared transmitting tube and infrared receiving tube are devices that convert electric energy into near-infrared light directly. Its structure and principle are similar to ordinary light-emitting diodes, but the semiconductor material is different.

The infrared receiver is a receiving, amplifying and demodulating device. The internal integrated circuit has been demodulated and the output is digital signal.

Infrared receiver         		Infrared transmitter        	Infrared receiver module

Working Principle

To understand the structure of infrared receiver: infrared receiver is composed of IC and PD. IC is the processing element of the receiver, mainly composed of silicon crystal and circuit. It is a highly integrated device. PD is a photodiode whose main function is to receive optical signals. The infrared emitting diode sends out the modulation signal, and the infrared receiving head recovers the signal after receiving, decoding, filtering and a series of operations.

Component List

RaspberryPi mainboard

Infrared transmitter *1

Infrared receiver*1

Several jumpers

接线

红外接收接线

| RaspberryPi | Infrared receiver |
| --- | --- |
| IO24(wiringPi)/19(BCM) | S |
| GND | - |
| +5V | + |

红外发送实验

| RaspberryPi | Infrared transmitter |
| --- | --- |
| IO1(wiringPi)/18(BCM) | S |
| GND | GND |

Code

Receiver

C++   main program

#include "IR_REC.h" int main() { int key; if(wiringPiSetup() == -1){ printf("setup wiringPi failed !"); return 1; } while(1){ key = GetKey(); if (key != ERROR) { printf("key: %x \n",key); } } }

Python program

#!/usr/bin/python # -*- coding:utf-8 -*- import RPi.GPIO as GPIO import time ERROR = 0xFE PIN = 19 #Define infrared receiver pin GPIO.setmode(GPIO.BCM) GPIO.setwarnings(False) GPIO.setup(PIN, GPIO.IN, GPIO.PUD_UP) #Set the infrared receiving pin to pull-up mode def getKey(): byte = [0, 0, 0, 0] if IRStart() == False: #Judging infrared guide pulse time.sleep(0.11) # One message frame lasts 108 ms. return ERROR else: for i in range(0, 4): byte[i] = getByte() # #Receive 32-bit infrared data (address, address inversion, data, data inversion) if byte[0] + byte[1] == 0xff and byte[2] + byte[3] == 0xff: # Check whether the received data is correct print("right") return byte[2] else: print("error") return ERROR #return byte[2] def IRStart(): timeFallingEdge = [0, 0] timeRisingEdge = 0 timeSpan = [0, 0] GPIO.wait_for_edge(PIN, GPIO.FALLING) timeFallingEdge[0] = time.time() GPIO.wait_for_edge(PIN, GPIO.RISING) timeRisingEdge = time.time() GPIO.wait_for_edge(PIN, GPIO.FALLING) timeFallingEdge[1] = time.time() timeSpan[0] = timeRisingEdge - timeFallingEdge[0] timeSpan[1] = timeFallingEdge[1] - timeRisingEdge print(timeSpan[0],timeSpan[1]) if timeSpan[0] > 0.0085 and timeSpan[0] < 0.0095 and timeSpan[1] > 0.004 and timeSpan[1] < 0.005: print("1") return True else: print("0") return False def getByte(): byte = 0 timeRisingEdge = 0 timeFallingEdge = 0 timeSpan = 0 for i in range(0, 8): GPIO.wait_for_edge(PIN, GPIO.RISING) timeRisingEdge = time.time() GPIO.wait_for_edge(PIN, GPIO.FALLING) timeFallingEdge = time.time() timeSpan = timeFallingEdge - timeRisingEdge if timeSpan > 0.0016 and timeSpan < 0.0018: byte ¦= 1 << i return byte print('IRM Test Start ...') try: while True: key = getKey() #Read infrared pulse if(key != ERROR): #Print infrared pulse value print("Get the key: 0x%02x" %key) except KeyboardInterrupt: GPIO.cleanup()

Java program

import com.pi4j.wiringpi.Gpio; public class IR_NEC { static int PIN = 24; static int ERROR = 0xfe, key; static long timeRisingEdge, timeFallingEdge, timeRising, timeFalling_0, timeFalling_1; static long timeSpan_val = 0; static long[] time_span = new long[2]; static { if (Gpio.wiringPiSetup() == -1) { System.out.println(" ==>> GPIO SETUP FAILED"); } Gpio.pinMode(PIN, Gpio.INPUT); } public static boolean IRStart() { while(!(Gpio.digitalRead(PIN) == 0)); timeFalling_0 = gettimeofday(); while(!(Gpio.digitalRead(PIN) == 1)); timeRising = gettimeofday(); while(!(Gpio.digitalRead(PIN) == 0)); timeFalling_1 = gettimeofday(); time_span[0] = timeRising - timeFalling_0; time_span[1] = timeFalling_1 - timeRising; // System.out.println("start_time " + time_span[0] + "," +time_span[1]); if (time_span[0] > 8500 && time_span[0] < 9500 && time_span[1] >= 4000 && time_span[1] <= 5000) { // System.out.println("start singe*************"); return true; } else { return false; } } public static long gettimeofday() { // return System.currentTimeMillis() ;// +System.nanoTime() / 1000; return System.nanoTime() / 1000; } public static int GetByte() { int byte_val = 0; for (int i = 0; i < 8; i++) { while(!(Gpio.digitalRead(PIN) == 1)); timeRisingEdge = gettimeofday(); while(!(Gpio.digitalRead(PIN) == 0)); timeFallingEdge = gettimeofday(); timeSpan_val = timeFallingEdge - timeRisingEdge; // System.out.print("start byte "); // System.out.println(timeSpan_val); if (timeSpan_val > 1500 && timeSpan_val < 1800) byte_val ¦= 1 << i; } // System.out.printf("byte_val: %x \n", byte_val); return byte_val; } public static int GetKey() { int[] byte_val = new int[4]; if (IRStart() == false) { Gpio.delay(108); return ERROR; } else { for (int i = 0; i < 4; i++) { byte_val[i] = GetByte(); // System.out.printf("byte_val[%d]: %x \n",i, byte_val[i]); } if ((byte_val[0] + byte_val[1] == 0xff) && (byte_val[2] + byte_val[3] == 0xff)) { return byte_val[2]; } else { return ERROR; } } } public static void main(String args[]) { int rec_val; IR_NEC ir_nec = new IR_NEC(); for ( ; ;) { rec_val = ir_nec.GetKey(); if (rec_val != ERROR) { System.out.printf("key: %x \n",rec_val); } } } }

Transmitter

C++ main program

#include "IR_SEND.h" int main() { if(wiringPiSetup() == -1){ printf("setup wiringPi failed !"); return 1; } while(1){ IR_Send(0x45); delay(200); } }

Python program

import ctypes import time so = ctypes.cdll.LoadLibrary lib = so("./libpycallclass.so") while True: lib.IR_Send(0x45) time.sleep(1)

Java program

import com.pi4j.wiringpi.Gpio; public class IR_SEND_OBJ { static int irsys = 0xfe, ircode; static int PIN = 1; public static void IR_Send_Start() { Gpio.pwmWrite(IR_SEND_OBJ.PIN,22); Gpio.delayMicroseconds(4500); Gpio.delayMicroseconds(4500); Gpio.pwmWrite(IR_SEND_OBJ.PIN,0); Gpio.delayMicroseconds(4500); } public static void Send_Byte(){ for (int i = 0; i < 8; i++) { if((IR_SEND_OBJ.ircode & 0x01) == 1) { Gpio.pwmWrite(IR_SEND_OBJ.PIN,22); Gpio.delayMicroseconds(560); Gpio.pwmWrite(IR_SEND_OBJ.PIN,0); Gpio.delayMicroseconds(1690); } else { Gpio.pwmWrite(IR_SEND_OBJ.PIN,22); Gpio.delayMicroseconds(560); Gpio.pwmWrite(IR_SEND_OBJ.PIN,0); Gpio.delayMicroseconds(560); } IR_SEND_OBJ.ircode = IR_SEND_OBJ.ircode >> 1; } } public static void IR_Send(int date) { Gpio.pinMode(IR_SEND_OBJ.PIN,Gpio.PWM_OUTPUT); Gpio.pwmSetMode(Gpio.PWM_MODE_MS); Gpio.pwmSetRange(45); Gpio.pwmSetClock(32); IR_SEND_OBJ.ircode = IR_SEND_OBJ.irsys; IR_SEND_OBJ.IR_Send_Start(); IR_SEND_OBJ.Send_Byte(); IR_SEND_OBJ.ircode = ~IR_SEND_OBJ.irsys; IR_SEND_OBJ.Send_Byte(); IR_SEND_OBJ.ircode = date; IR_SEND_OBJ.Send_Byte(); IR_SEND_OBJ.ircode = ~date; IR_SEND_OBJ.Send_Byte(); Gpio.pwmWrite(IR_SEND_OBJ.PIN,22); Gpio.delayMicroseconds(400); Gpio.pwmWrite(IR_SEND_OBJ.PIN,0); } public static void main(String args[]) { // setup wiring pi if (Gpio.wiringPiSetup() == -1) { System.out.println(" ==>> GPIO SETUP FAILED"); } while (true){ IR_SEND_OBJ.IR_Send(0x45); Gpio.delay(1000); } } }

Experiment Result